Staged Rollout Report

The Early Adopter team should fill the “grey” areas

| NGI | NGI-DE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Site Name | LRZ |  |  |  |  |  |  |
| EA team names | Matteo Lanati |  |  |  |  |  |  |
| EA team contacts | matteo.lanati@lrz.de |  |  |  |  |  |  |
| Product | Globus GridFTP | OS | SL6 | Version | 10.4.1 | RT ticket ID | 11292 |

The “outcome” is one of: OK, WARN, FAIL, NA (Not Aplicable), Nver (Not verified/Not tested).

Copy/paste any results from your commands where you see fit.

| Metric | <OK¦WARN¦FAIL ¦NA¦NVer> | WEB URLs, Command used, other comments (please always fill this table with as much information as possible, and as you see fit) |
| --- | --- | --- |
| Release notes and other documentation | OK | The file /usr/share/doc/globus-gridftp-server-10.4/README contains all the links to the online documentation. |
| Installation or upgrading (specify which) | OK | Upgrading. |
| (RE-)configuration (if using yaim, specify command used) | OK | The configuration file(s) didn’t change. |
| Functionality (including start stop of deamons, and which) | OK | /etc/init.d/globus-gridftp-server offers the standard options (start¦stop¦status¦restart¦reload¦try-restart¦force-reload). The behaviour is correct, but (on SL 6.3): - in case the “start” command is issued more than one time, the output does not specify that the service is already running - in case the “stop” command is issued more than one time, the output does not specify that the service is not running (but the script complains about the missing lock file). |
| SAM/Nagios/Gstat or any other monitoring framework (specify the name of the machine) | NA | Probes are not provided. |
| Interaction integration with other components (specify which) | OK | Globus core libraries. |
| Behaviour in production environment (after a few days). Specify VO’s using the service when applicable | OK | No major problems encountered. |
| GGUS tickets opened: please insert the URLs of the tickets | NA |  |
| Log files: usefulness, clarity, etc., any other comments on where/how to improve them. | OK |  |
| Error messages: please specify or comment any issues with error messages, if they are not correct, not clear, useful, etc. . | NA |  |

globus-url-copy -list gsiftp://vm-10-155-208-132.cloud.mwn.de/tmp/

gsiftp://vm-10-155-208-132.cloud.mwn.de/tmp/

.ICE-unix/

x509up_p21978.fileXtKuoQ.1

x509up_p21981.filerRSCAw.1…

macbookml:~ lu95jib$ globus-url-copy -vb -t 10 gsiftp://vm-10-155-208-132.cloud.mwn.de/dev/zero /dev/null

Source: gsiftp://vm-10-155-208-132.cloud.mwn.de/dev/

Dest:   file:///dev/

zero  ->  null

910163968 bytes       108.50 MB/sec avg       112.50 MB/sec inst

Cancelling copy...

macbookml:~ lu95jib$ globus-url-copy -vb -t 10 /dev/zero gsiftp://vm-10-155-208-132.cloud.mwn.de/dev/null

Source: file:///dev/

Dest:   gsiftp://vm-10-155-208-132.cloud.mwn.de/dev/

zero  ->  null

1052770304 bytes       108.25 MB/sec avg       113.00 MB/sec inst

Cancelling copy...

macbookml:~ lu95jib$ globus-url-copy -vb -r -cd Documents/EGI/ gsiftp://vm-10-155-208-132.cloud.mwn.de/tmp/EGI/

Source: file:///Users/lu95jib/Documents/EGI/2015/early_adopters_cert_sl6_20150220/

Dest:   gsiftp://vm-10-155-208-132.cloud.mwn.de/tmp/EGI/2015/early_adopters_cert_sl6_20150220/

EGI-TSA1.3-staged-rollout-gridftp-7.18-1.doc

Source: file:///Users/lu95jib/Documents/EGI/2015/early_adopters_cert_sl6_20150220/

Dest:   gsiftp://vm-10-155-208-132.cloud.mwn.de/tmp/EGI/2015/early_adopters_cert_sl6_20150220/

EGI-TSA1.3-staged-rollout-myproxy-6.1.10.doc…

macbookml:~ lu95jib$ globus-url-copy -list gsiftp://vm-10-155-208-132.cloud.mwn.de/tmp/EGI/

gsiftp://vm-10-155-208-132.cloud.mwn.de/tmp/EGI/

2011/

2012/

2013/

…